Table 1. Electronic system and circuit components of the simulated sensor interface.

| Block | Model / parameters (simulation) | Role in the self-test | Source (code constant) |
| --- | --- | --- | --- |
| Sensor model | baseline U(0.78,1.18) V; carrier amplitude U(0.045,0.18) V; frequency U(0.45,3.0) Hz; sensor noise U(2,14) mV; slow component 25 mV at 0.12 Hz; random-walk drift 1.5 mV/step | Physical signal source; replaced by R1 during the AFE test window | src/simulate.py: make_condition, _base_signal |
| MUX 1 / reference R1 | 140-260 ms; 0.90 V then 1.30 V (two 60 ms plateaus); replaces the sensor at the AFE input | Two-level reference at the AFE input; its response passes through AFE and ADC | src/simulate.py: _afe_reference |
| AFE / filter | AFE gain U(1.65,2.05); AFE tau U(8,16) ms; first-order RC low-pass with gain and offset | Amplifies and filters; faults: offset, bandwidth | src/simulate.py: make_condition, _base_signal |
| MUX 2 / reference R2 | 340-460 ms; 1.15 V then 1.75 V; bypasses the AFE at the ADC input | Two-level reference at the ADC input, bypassing the AFE; isolates the ADC transfer | src/simulate.py: _adc_reference |
| ADC | 1 kHz, 512 samples per record (0.512 s), 12-bit ADC over 0-3.3 V, ADC noise 2.5 mV rms | Digitises all paths; faults: offset, gain, clipping | src/simulate.py: simulate_sample |
| Reference monitor | buffered 12-bit channel, 16-sample mean, 0.25 mV rms after averaging, common gain error U(-E,E), E in {0,1,2,5}% | Measures the actual reference levels used in calibration (simulated, not built) | src/monitor_model.py |
| Switches (ngspice) | Ron = 0.5 Ohm, Roff = 1 MOhm, threshold 2.5 V, hysteresis 0.1 V; 1 us edges | Circuit-level multiplexer model | src/spice_phase.py: render_phase |
| Extra ngspice noise | 0.5 mV rms amplifier-input noise and 0.5 mV rms on each reference rail | Circuit-domain noise not present in the behavioral model | src/spice_domain.py: AMP_NOISE_V, REF_NOISE_V |
| Digital classifier | MLP 20-96-48-{4 or 9}, ReLU, Adam, early stopping; 6,868 (layer) / 7,113 (fault) parameters; input: 20 (15 settled-plateau / calibration / sensor-domain + 5 phase-robust transient) | Classifies faulty layer (4 classes) and fault (9 classes) | phase_robust_results/timing_and_parameters.json; src/run_feasibility.py: _classifier |
| Component tolerances (test) | AFE gain ±5%, AFE tau ±10%, ADC gain ±2%, ADC offset ±10 mV, sensor baseline ±5% (uniform per sample) | Applied in both domains, drawn per sample | src/run_reference_monitor.py: _draw_tol |

Values are code constants copied programmatically from paper_package/PAPER_TABLES.csv (Table T1/T10); the simulator source code is not part of the evidence package, so these constants cannot be re-verified here.

Table 2. Fault taxonomy: nine fault classes and their physical location.

| # | Fault class | Layer | Physical location | Severity range (simulated) | Primary observability |
| --- | --- | --- | --- | --- | --- |
| 0 | Healthy | — | — | no fault injected | — |
| 1 | Sensor bias | Sensor | Sensor element / sensor output | ±U(0.06,0.24) V | Passive sensor segments only (references replace the sensor) |
| 2 | Sensor gain | Sensor | Sensor element (sensitivity) | ±U(6,28)% | Passive sensor segments only (references replace the sensor) |
| 3 | Sensor noise | Sensor | Sensor element (excess noise) | U(25,95) mV rms | Passive sensor segments only (references replace the sensor) |
| 4 | AFE offset | AFE | Amplifier stage (offset) | ±U(0.06,0.24) V | R1 window, after removing the ADC transfer from R2 |
| 5 | AFE bandwidth | AFE | RC low-pass filter (time constant τ) | tau x U(1.8,4.5) | R1 window, after removing the ADC transfer from R2 |
| 6 | ADC offset | ADC | ADC (offset) | ±U(0.06,0.24) V | R2 window (bypasses the AFE) |
| 7 | ADC gain | ADC | ADC (gain) | ±U(6,28)% | R2 window (bypasses the AFE) |
| 8 | ADC clipping | ADC | ADC input range (clipping) | full scale x U(0.72,0.93) | R2 window (bypasses the AFE) |

Severity ranges parsed programmatically from the 'Faults (severity)' row of paper_package/PAPER_TABLES.csv (src/simulate.py: make_fault). Layer assignment follows the four-class layer label used by the classifier.

Table 3. Simulation and validation protocol.

| Item | Setting |
| --- | --- |
| Simulator domains | Behavioral discrete-time model; ngspice per-sample netlist of the same circuit |
| Dataset seeds | 20261026, 20261027, 20261028 (3 seeds, unseen before the protocol freeze) |
| Conditions per seed | 350 operating conditions × 9 fault classes |
| Train / validation / test split | 222 / 40 / 88 conditions per seed; overlap 0 |
| Test records per test cell | 792 (88 conditions × 9 classes) |
| Edge phases ψ (test) | 0, 0.25, 0.5, 0.75, 0.999 (5 phases); training ψ ~ U[0,1) for randomized arms |
| Reference amplitudes | Behavioral ×0.8, ×0.9, ×1, ×1.1, ×1.2; ngspice ×0.8, ×1, ×1.2 |
| Monitor gain-error bound E | 0%, ±1%, ±2%, ±5% (matched training) and known reference |
| Component tolerances | AFE gain ±5%, AFE tau ±10%, ADC gain ±2%, ADC offset ±10 mV, sensor baseline ±5% (uniform per sample) |
| Noise | ADC noise 2.5 mV rms; ngspice adds 0.5 mV rms amplifier-input noise and 0.5 mV rms on each reference rail |
| Quantization | 12-bit ADC over 0-3.3 V |
| ngspice runs | 13,878 per seed; 41,634 in total |
| Initializations | Two initialization seeds per model, averaged |
| Primary metric | Layer macro-F1 (4 classes); also fault macro-F1 (9 classes), balanced accuracy, per-class recall |
| Statistics | Paired condition-level cluster bootstrap, 5000 resamples per seed; pooled CI of the mean of the three per-seed differences |
| Pre-registration | Protocol, descriptors, arms, thresholds and decision rule frozen before validation data (source hashes recorded) |

Sources: seed_*/meta.json, PROTOCOL_PHASE_ROBUST_FROZEN.md, paper_package/PAPER_TABLES.csv (T1).

Table 4. Main classification results of the primary (PRr) and secondary (PRrM) methods.

| Method | Role | Domain | Layer macro-F1 | Layer balanced acc. | Fault macro-F1 | Fault balanced acc. |
| --- | --- | --- | --- | --- | --- | --- |
| PRr | Primary (pre-registered) | Behavioral | 0.656 | 0.660 | 0.668 | 0.670 |
| PRr | Primary (pre-registered) | ngspice | 0.649 | 0.654 | 0.662 | 0.665 |
| PRrM | Secondary (mixed-domain training) | Behavioral | 0.672 | 0.674 | 0.675 | 0.676 |
| PRrM | Secondary (mixed-domain training) | ngspice | 0.663 | 0.666 | 0.671 | 0.673 |

Monitor E = ±2%, pooled over amplitudes (behavioral ×0.8–×1.2; ngspice ×0.8, ×1.0, ×1.2) and the five phases; mean of 3 dataset seeds × 2 initializations. The behavioral and ngspice rows use different amplitude grids and must not be subtracted; the matched-grid transfer comparison is in Table 9. Nominal reference value N (known reference, ×1.0, pooled over phases): 0.659 behavioral, 0.660 ngspice. Source: attribution_arms.csv; decision_rules.json (N).

Table 5. Pre-registered gate results for the primary method PRr.

| Gate | Criterion | Threshold | Observed (PRr) | 95% CI | Behavioral | ngspice |
| --- | --- | --- | --- | --- | --- | --- |
| A | Retention at ×0.8 and ×1.2 (known, E ≤ ±2%) | ≥ 0.85·N | worst 0.916 (ngspice, monitor E = ±1%, ×0.8) | — | PASS | PASS |
| B | Superiority over old method and one-point baselines (behavioral) | pooled CI lower bound > 0 | smallest lower bound +0.112 over 70 comparisons | per comparison, Table 6 | PASS | — |
| C | AFE–ADC cross-confusion at ×0.8 and ×1.2 | < 0.15 | largest 0.089 (behavioral, known reference, ×1.2) | — | PASS | PASS |
| D | Retention with monitor E = ±5% | ≥ 0.70·N | worst 0.924 (ngspice, ×0.8) | — | PASS | PASS |
| E | Behavioral → ngspice transfer loss (E = ±2%, ngspice grid) | ≤ 0.10 (point estimate) | 0.002 | [-0.003, +0.006] | PASS | — |
| F | Every claimed improvement supported by a paired-bootstrap CI excluding zero | CI excludes 0 | satisfied | 5000 resamples | procedural† | procedural† |
| G | Layer macro-F1 range over five phases (known, E = ±2%) | ≤ 0.05 | largest 0.011 (smallest 0.005) | — | PASS | PASS |

Decision: GO (0 failed gates). N = PRr layer macro-F1 with known reference at ×1.0 (0.659 behavioral, 0.660 ngspice). †Gate F has no separate key in decision_rules.json; it was checked here against the saved CIs. The mixed-domain variant PRrM also passes the ngspice-domain gates (reported only). Source: decision_rules.json (gates, A–E, G details); PROTOCOL_PHASE_ROBUST_FROZEN.md §5 (thresholds).

Table 6. Baseline comparison (layer macro-F1, behavioral domain).

| Method | Layer macro-F1 ×0.8 | Layer macro-F1 ×1.2 | PRr − method ×0.8 [95% CI] | PRr − method ×1.2 [95% CI] | PRr − method, E = ±2% pooled [95% CI] |
| --- | --- | --- | --- | --- | --- |
| Passive (sensor signal only), old descriptors | 0.259 | 0.259 | +0.356 [+0.341, +0.371] | +0.384 [+0.368, +0.400] | — |
| Single-probe active test, old descriptors | 0.349 | 0.396 | +0.266 [+0.250, +0.282] | +0.246 [+0.230, +0.261] | — |
| AFE-only reference, old descriptors | 0.320 | 0.295 | +0.295 [+0.280, +0.311] | +0.348 [+0.329, +0.366] | — |
| ADC-only reference, old descriptors | 0.120 | 0.122 | +0.495 [+0.482, +0.508] | +0.521 [+0.507, +0.534] | — |
| A0: two-point, old descriptors (previous method) | 0.140 | 0.121 | +0.475 [+0.461, +0.489] | +0.521 [+0.507, +0.535] | +0.375 [+0.363, +0.387] |
| AFE-only reference, PRr descriptors | 0.473 | 0.516 | +0.142 [+0.128, +0.155] | +0.126 [+0.112, +0.140] | — |
| ADC-only reference, PRr descriptors | 0.455 | 0.452 | +0.160 [+0.144, +0.175] | +0.190 [+0.176, +0.206] | — |
| R1: Revision-1 fixed-sample descriptors | 0.578 | 0.593 | — | — | +0.042 [+0.034, +0.050] |
| R1r: R1 + phase-randomized training | 0.626 | 0.634 | — | — | +0.012 [+0.004, +0.019] |
| PR: phase-robust descriptors, ψ = 0 training | 0.612 | 0.635 | — | — | +0.000 [-0.005, +0.006] |
| PRr: proposed (primary) | 0.615 | 0.643 | — | — | — |
| PRrM: PRr + mixed-domain training (secondary) | 0.660 | 0.670 | — | — | -0.017 [-0.025, -0.008] |

Behavioral domain, known reference, pooled over the five phases. Absolute layer macro-F1 values are computed from the saved per-sample predictions (they are not stored in a summary file); for every baseline the difference PRr − method equals the saved paired difference in decision_rules.json (B) to within 1e-9. Paired differences and CIs: decision_rules.json (B) and attribution_comparisons.csv; PRr − PRrM is the sign-reversed PRrM − PRr comparison (CI bounds reversed accordingly).

Table 7. Reference-monitor robustness: layer macro-F1 versus actual reference amplitude.

| Reference condition | Behavioral ×0.8 | Behavioral ×0.9 | Behavioral ×1.0 | Behavioral ×1.1 | Behavioral ×1.2 | ngspice ×0.8 | ngspice ×1.0 | ngspice ×1.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Known reference | 0.615 | 0.645 | 0.658 | 0.667 | 0.642 | 0.616 | 0.660 | 0.642 |
| Monitor E = 0% | 0.610 | 0.638 | 0.655 | 0.665 | 0.642 | 0.606 | 0.655 | 0.644 |
| Monitor E = ±1% | 0.608 | 0.638 | 0.658 | 0.670 | 0.644 | 0.604 | 0.655 | 0.644 |
| Monitor E = ±2% | 0.614 | 0.647 | 0.673 | 0.681 | 0.663 | 0.611 | 0.671 | 0.664 |
| Monitor E = ±5% | 0.611 | 0.644 | 0.661 | 0.676 | 0.649 | 0.610 | 0.663 | 0.646 |
| No-monitor control (nominal levels assumed) | 0.310 | 0.423 | 0.659 | 0.389 | 0.237 | — | — | — |

PRr layer macro-F1 pooled over the five phases (mean of three seeds). Monitored variants are trained with the matching error bound E. The no-monitor control is the PRr known-reference classifier given the nominal levels; it was evaluated in the behavioral domain only. Retention relative to N: worst 0.916 at ±20% (known, E ≤ ±2%) and 0.924 at E = ±5%. Source: amplitude_curves.csv; no_monitor_control.json; decision_rules.json (A, D).

Table 8. Phase-robustness ablation: layer macro-F1 per edge phase.

| Domain | Arm | ψ = 0 | ψ = 0.25 | ψ = 0.5 | ψ = 0.75 | ψ = 0.999 | Range (max − min) | ≤ 0.05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Behavioral | A0 | 0.279 | 0.279 | 0.281 | 0.281 | 0.282 | 0.003 | yes |
| Behavioral | R1 | 0.649 | 0.636 | 0.614 | 0.590 | 0.568 | 0.081 | no |
| Behavioral | R1r | 0.651 | 0.650 | 0.648 | 0.641 | 0.626 | 0.025 | yes |
| Behavioral | PR | 0.657 | 0.656 | 0.655 | 0.655 | 0.653 | 0.004 | yes |
| Behavioral | PRr | 0.659 | 0.658 | 0.656 | 0.653 | 0.652 | 0.006 | yes |
| Behavioral | PRrM | 0.677 | 0.676 | 0.672 | 0.668 | 0.667 | 0.010 | yes |
| ngspice | R1 | 0.642 | 0.625 | 0.606 | 0.584 | 0.561 | 0.081 | no |
| ngspice | R1r | 0.643 | 0.642 | 0.638 | 0.631 | 0.617 | 0.026 | yes |
| ngspice | PR | 0.649 | 0.648 | 0.647 | 0.644 | 0.642 | 0.007 | yes |
| ngspice | PRr | 0.654 | 0.651 | 0.649 | 0.645 | 0.643 | 0.010 | yes |
| ngspice | PRrM | 0.667 | 0.665 | 0.664 | 0.659 | 0.658 | 0.009 | yes |

Layer macro-F1 per edge phase, monitor E = ±2%, pooled over amplitudes; range computed as max − min of the five stored phase values. Gate G itself applies to PRr (known reference and E = ±2%; Table 5): PRr ranges 0.005 / 0.006 (behavioral known / E2) and 0.011 / 0.010 (ngspice). A0 was not simulated in ngspice; its flat profile reflects uniformly low accuracy. Source: attribution_arms.csv; decision_rules.json (G).

Table 9. Behavioral-to-ngspice domain transfer (layer macro-F1).

| Reference condition | Behavioral BB (PRr) | ngspice BS (PRr) | Transfer loss BB − BS | 95% CI | ≤ 0.10 | ngspice MS (PRrM) | MS − BS [95% CI] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Known reference | 0.639 | 0.639 | -0.001 | [-0.005, 0.004] | PASS | 0.677 | +0.038 [+0.029, +0.047] |
| Monitor E = 0% | 0.636 | 0.635 | 0.001 | [-0.003, 0.004] | PASS | 0.675 | +0.040 [+0.031, +0.050] |
| Monitor E = ±1% | 0.637 | 0.634 | 0.002 | [-0.001, 0.006] | PASS | 0.670 | +0.036 [+0.027, +0.045] |
| Monitor E = ±2% | 0.650 | 0.649 | 0.002 | [-0.003, 0.006] | PASS | 0.663 | +0.014 [+0.005, +0.024] |
| Monitor E = ±5% | 0.640 | 0.640 | 0.001 | [-0.003, 0.005] | PASS | 0.655 | +0.015 [+0.006, +0.024] |
| Previous stage, E = ±2% (REVISE) | 0.658 | 0.539 | 0.119 | [0.107, 0.132] | FAIL | — | — |

Matched grid: ×0.8, ×1.0, ×1.2 × five phases in both domains. BB and BS use the same behavioral-trained PRr classifier; MS is the mixed-trained secondary variant PRrM. The pre-registered gate E uses E = ±2%. The previous-stage row is the failed earlier validation (reference_monitor_final), read from paper_package/NUMBER_PROVENANCE.csv because its decision file is not in the evidence package. Source: decision_rules.json (E).

Table 10. AFE/ADC fault separation of PRr at ±20% reference amplitude.

| Domain | Reference / monitor condition | Amplitude | AFE recall | ADC recall | AFE–ADC cross-confusion |
| --- | --- | --- | --- | --- | --- |
| Behavioral | Known reference | ×0.8 | 0.912 | 0.831 | 0.026 |
| Behavioral | Known reference | ×1.2 | 0.898 | 0.797 | 0.089 |
| Behavioral | Monitor E = 0% | ×0.8 | 0.883 | 0.843 | 0.035 |
| Behavioral | Monitor E = 0% | ×1.2 | 0.865 | 0.797 | 0.085 |
| Behavioral | Monitor E = ±1% | ×0.8 | 0.905 | 0.833 | 0.032 |
| Behavioral | Monitor E = ±1% | ×1.2 | 0.876 | 0.790 | 0.087 |
| Behavioral | Monitor E = ±2% | ×0.8 | 0.886 | 0.826 | 0.035 |
| Behavioral | Monitor E = ±2% | ×1.2 | 0.885 | 0.797 | 0.079 |
| Behavioral | Monitor E = ±5% | ×0.8 | 0.891 | 0.778 | 0.033 |
| Behavioral | Monitor E = ±5% | ×1.2 | 0.878 | 0.777 | 0.083 |
| ngspice | Known reference | ×0.8 | 0.909 | 0.833 | 0.028 |
| ngspice | Known reference | ×1.2 | 0.898 | 0.799 | 0.088 |
| ngspice | Monitor E = 0% | ×0.8 | 0.879 | 0.845 | 0.036 |
| ngspice | Monitor E = 0% | ×1.2 | 0.863 | 0.800 | 0.083 |
| ngspice | Monitor E = ±1% | ×0.8 | 0.899 | 0.835 | 0.033 |
| ngspice | Monitor E = ±1% | ×1.2 | 0.874 | 0.792 | 0.086 |
| ngspice | Monitor E = ±2% | ×0.8 | 0.883 | 0.832 | 0.036 |
| ngspice | Monitor E = ±2% | ×1.2 | 0.885 | 0.797 | 0.078 |
| ngspice | Monitor E = ±5% | ×0.8 | 0.887 | 0.773 | 0.034 |
| ngspice | Monitor E = ±5% | ×1.2 | 0.875 | 0.778 | 0.081 |
| Behavioral | No monitor (nominal levels assumed) | ×0.8 | 0.426 | 0.944 | 0.230 |
| Behavioral | No monitor (nominal levels assumed) | ×1.2 | 0.375 | 0.820 | 0.336 |

PRr, layer classifier, pooled over the five phases (mean of three seeds × two initializations). Cross-confusion = fraction of AFE- and ADC-fault records assigned to the other of the two layers (gate C limit 0.15). Source: decision_rules.json (C); no-monitor rows computed from seed_*/predictions.npz with the same definition (the computation reproduces every saved C value exactly; see verification/RECOMPUTED_FROM_PREDICTIONS.csv).

Table 11. Per-class recall (fine-fault and layer classes).

| Domain | Arm | Healthy | Sensor bias | Sensor gain | Sensor noise | AFE offset | AFE bandwidth | ADC offset | ADC gain | ADC clipping | Layer: Healthy | Layer: Sensor | Layer: AFE | Layer: ADC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Behavioral | A0 | 0.015 | 0.052 | 0.057 | 0.094 | 0.135 | 0.511 | 0.539 | 0.800 | 0.057 | 0.010 | 0.154 | 0.238 | 0.882 |
| Behavioral | R1 | 0.230 | 0.231 | 0.312 | 0.988 | 0.763 | 0.938 | 0.992 | 0.948 | 0.398 | 0.098 | 0.703 | 0.866 | 0.822 |
| Behavioral | R1r | 0.328 | 0.316 | 0.261 | 0.985 | 0.823 | 0.929 | 0.997 | 0.957 | 0.311 | 0.148 | 0.804 | 0.873 | 0.764 |
| Behavioral | PR | 0.276 | 0.279 | 0.323 | 0.983 | 0.855 | 0.955 | 0.995 | 0.918 | 0.429 | 0.132 | 0.782 | 0.927 | 0.803 |
| Behavioral | PRr | 0.294 | 0.260 | 0.314 | 0.984 | 0.857 | 0.955 | 0.997 | 0.922 | 0.447 | 0.142 | 0.780 | 0.911 | 0.806 |
| Behavioral | PRrM | 0.355 | 0.256 | 0.338 | 0.983 | 0.921 | 0.942 | 0.997 | 0.927 | 0.368 | 0.238 | 0.742 | 0.940 | 0.775 |
| ngspice | R1 | 0.229 | 0.235 | 0.312 | 0.988 | 0.755 | 0.918 | 0.990 | 0.938 | 0.382 | 0.094 | 0.694 | 0.860 | 0.815 |
| ngspice | R1r | 0.310 | 0.326 | 0.264 | 0.986 | 0.814 | 0.910 | 0.996 | 0.947 | 0.304 | 0.143 | 0.795 | 0.867 | 0.756 |
| ngspice | PR | 0.247 | 0.284 | 0.325 | 0.983 | 0.839 | 0.942 | 0.993 | 0.901 | 0.455 | 0.118 | 0.775 | 0.920 | 0.803 |
| ngspice | PRr | 0.266 | 0.263 | 0.310 | 0.982 | 0.845 | 0.942 | 0.995 | 0.907 | 0.475 | 0.134 | 0.773 | 0.901 | 0.810 |
| ngspice | PRrM | 0.320 | 0.256 | 0.347 | 0.983 | 0.912 | 0.928 | 0.995 | 0.915 | 0.399 | 0.220 | 0.735 | 0.933 | 0.776 |
| Behavioral | PRr, no monitor (×0.8/×1.2) | — | — | — | — | — | — | — | — | — | 0.000 | 0.039 | 0.400 | 0.882 |

Recall, monitor E = ±2%, pooled over amplitudes and phases (mean of three seeds). Weak classes (recall < 0.5) for PRr: healthy, sensor bias, sensor gain and ADC clipping; healthy-layer recall 0.142 (behavioral) / 0.134 (ngspice). Source: per_class_recall_fault.csv; per_class_recall_layer.csv.

Table 12. Model and computational complexity.

| Quantity | Value | Source |
| --- | --- | --- |
| MLP architecture | 20–96–48 hidden layout; 20 inputs; ReLU; output 4 (layer) or 9 (fault) | PAPER_TABLES.csv T10; parameter count check |
| MLP parameters | 6,868 (layer head); 7,113 (fault head) | timing_and_parameters.json |
| Multiply-accumulates per inference | 6,720 (layer); 6,960 (fault) | derived from the architecture |
| MLP inference time | 0.68 µs per record (batched, Python, mean of 3 seeds) | timing_and_parameters.json |
| Descriptor extraction time | Phase-robust 0.79 ms; Revision-1 0.47 ms per record (Python, 1 core) | timing_and_parameters.json |
| MLP training time (PRr models) | mean 0.72 s, max 1.49 s per model | seed_*/model_info.json |
| ngspice runs | 13,878 per seed; 41,634 total | seed_*/meta.json |
| Validation runtime per seed | 20261026: total 1175.6 s (ngspice wall 908.1 s, behavioral 707.7 s); 20261027: total 1179.0 s (ngspice wall 908.9 s, behavioral 708.5 s); 20261028: total 1179.1 s (ngspice wall 905.9 s, behavioral 703.8 s) | seed_*/meta.json |
| ngspice workers | 3 parallel workers | timing_and_parameters.json |
| Important assumptions | Timings are desktop Python (macOS arm64) measurements, not microcontroller measurements; no energy, power, area or hardware cost was measured | REPRODUCIBILITY_CHECKLIST.md (environment) |

No embedded-target timing or energy figure exists in the evidence package; none is claimed.