Table 10. AFE/ADC fault separation of PRr at ±20% reference amplitude.

| Domain | Reference / monitor condition | Amplitude | AFE recall | ADC recall | AFE–ADC cross-confusion |
| --- | --- | --- | --- | --- | --- |
| Behavioral | Known reference | ×0.8 | 0.912 | 0.831 | 0.026 |
| Behavioral | Known reference | ×1.2 | 0.898 | 0.797 | 0.089 |
| Behavioral | Monitor E = 0% | ×0.8 | 0.883 | 0.843 | 0.035 |
| Behavioral | Monitor E = 0% | ×1.2 | 0.865 | 0.797 | 0.085 |
| Behavioral | Monitor E = ±1% | ×0.8 | 0.905 | 0.833 | 0.032 |
| Behavioral | Monitor E = ±1% | ×1.2 | 0.876 | 0.790 | 0.087 |
| Behavioral | Monitor E = ±2% | ×0.8 | 0.886 | 0.826 | 0.035 |
| Behavioral | Monitor E = ±2% | ×1.2 | 0.885 | 0.797 | 0.079 |
| Behavioral | Monitor E = ±5% | ×0.8 | 0.891 | 0.778 | 0.033 |
| Behavioral | Monitor E = ±5% | ×1.2 | 0.878 | 0.777 | 0.083 |
| ngspice | Known reference | ×0.8 | 0.909 | 0.833 | 0.028 |
| ngspice | Known reference | ×1.2 | 0.898 | 0.799 | 0.088 |
| ngspice | Monitor E = 0% | ×0.8 | 0.879 | 0.845 | 0.036 |
| ngspice | Monitor E = 0% | ×1.2 | 0.863 | 0.800 | 0.083 |
| ngspice | Monitor E = ±1% | ×0.8 | 0.899 | 0.835 | 0.033 |
| ngspice | Monitor E = ±1% | ×1.2 | 0.874 | 0.792 | 0.086 |
| ngspice | Monitor E = ±2% | ×0.8 | 0.883 | 0.832 | 0.036 |
| ngspice | Monitor E = ±2% | ×1.2 | 0.885 | 0.797 | 0.078 |
| ngspice | Monitor E = ±5% | ×0.8 | 0.887 | 0.773 | 0.034 |
| ngspice | Monitor E = ±5% | ×1.2 | 0.875 | 0.778 | 0.081 |
| Behavioral | No monitor (nominal levels assumed) | ×0.8 | 0.426 | 0.944 | 0.230 |
| Behavioral | No monitor (nominal levels assumed) | ×1.2 | 0.375 | 0.820 | 0.336 |

PRr, layer classifier, pooled over the five phases (mean of three seeds × two initializations). Cross-confusion = fraction of AFE- and ADC-fault records assigned to the other of the two layers (gate C limit 0.15). Source: decision_rules.json (C); no-monitor rows computed from seed_*/predictions.npz with the same definition (the computation reproduces every saved C value exactly; see verification/RECOMPUTED_FROM_PREDICTIONS.csv).